Notes on Using the Ohio Phosphorus Index with MMP

Last Revised: 21-Feb-2005

(1)  The Ohio Phosphorus Index is a custom tool for MMP that automatically analyzes phosphorus loss risk factors.  This analysis is based on the Nitrogen and Phosphorus Transport Risk Assessment Procedures from Section 1 of the Ohio NRCS Field Office Technical Guide (http://www.nrcs.usda.gov/technical/efotg/), revised January 2001.

(2)  MMP's Phosphorus Index tool requires Microsoft Access, version 2000 or later.

(3)  Be sure to enter each field's estimated sheet and rill erosion in the Water Erosion column on MMP's Assessment panel.  With RUSLE2, this is the "Soil loss for cons. plan" value.

(4)  Be sure to enter each field's distance to water in the Distance To Water column and indicate whether the field is drained on MMP's Assessment panel.

(5)  If the field’s slope is outside of the slope range for the predominant soil type, enter the actual slope on the Fields panel.

(6)  Application method factors represent an average of all applications during the given crop year.

(7)  Crop residue coverage at time of application (commercial or organic) is estimated based on the planned application timing and crop rotation.